3	Binary Adder

In this experiment, the student will become familiar with the operation of a 4-bit binary adder, and will also learn how to cascade two 4-bit binary adders to form an 8-bit binary adder.

I. 4-Bit Binary adder

The TA will program one of your 22V10 Programmable logic devices so  that it implements a 4-bit binary adder function.  The pinout for this device is:

- Pin 12 – GND,  Pin 24  -  Vcc.

- Four bit A input:    A(3): pin2, A(2): pin3, A(1): pin4, A(0): pin5

- Four bit B input      B(3): pin6, B(2): pin7, B(1): pin8, B(0): pin9

- Carry-In :  pin 10

- Four Bit Sum output:  Sum(3): pin22, Sum(2): pin21, Sum(1): pin20, Sum(0): pin19

- Carry Out:  pin 18

Unused inputs can be left unconnected.

The logic symbol for a 4 bit binary adder is shown below.  A(0) is the LSB (least significant bit) of A, A(3) is the MSB (Most Significant Bit) of A.

Cin

A(3)

A(2)

A(1)

A(0)

B(3)

B(2)

B(1)

B(0)

Sum(3)

Sum(2)

Sum(1)

Sum(0)

Cout

4 Bit Adder

A[3:0]

B[3:0]

Cin

Sum[3:0]

4

4

4

Cout

4 Bit Adder

Bus notation

- Connect the 4 bit  Sum output of the adder to LEDs.  Connect COUT output to an LED.  Connect the two 4 bit inputs A[3:0], B[3:0] to switch inputs.  Use a wire connection to either GND or VCC for the CIN input as appropriate for testing.

- Fill in Table #1 in the lab data sheet for your circuit.

II. 8 Bit Binary adder

- Connect two of the 4-bit binary adder chips to form an 8-bit adder. Use the diagram below to guide you (the carry out of the first 4-bit adder becomes the carry-in of the 2nd four-bit adder).

A[3:0]

B[3:0]

Cin

Sum[3:0]

4

4

4

Cout

8 Bit Adder from two 4-bit Adders

Bus notation is used in schematic

A[3:0]

B[3:0]

Cin

Sum[3:0]

4

4

4

Cout

8

A[7:0]

A[3:0]

A[7:4]

B[7:0]

8

B[3:0]

B[7:4]

Sum[3:0]

Sum[7:4]

8

Sum[7:0]

Tie Cin to GND.  Tie 8 switch inputs to the A input (bits 7 down to 0).  Use connections to +5 v/ GND for the B inputs.

- Verify the operation of the 8 bit adder by applying the test inputs below (values are in DECIMAL).  Fill in Table #2 of the lab data sheet.

b)    75
 +  (-22)
-----------

a)    56
 +    23
-----------

c)    -53
 +  (-42)
-----------

d)    -65
 +  (-80)
-----------

Two’s complement overflow could be a problem for some of the above additions. Which ones overflow the 8-bit range for two’s complement?

PRELAB Data page                TA Checkoff________

- Fill in the following table (Table #1)  for the operation of the 4-bit adder.

| Cin | A3 | A2 | A1 | A0 | B3 | B2 | B1 | B0 | S3 | S2 | S1 | S0 | Co |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |

- Perform the binary addition operations in section III – 8 bit additions

Examples:

| Decimal | Binary |  | Decimal | Binary |
| --- | --- | --- | --- | --- |
| 36 | 00100100 |  | 53 | 00110101 |
| (+) 15 | 00001111 |  | (-) 19 | 11101101 |
| 51 | 00110011 |  | 34 | 00100010 |

Fill in the following values:

| Decimal | Binary |  | Decimal | Binary |
| --- | --- | --- | --- | --- |
| 56 |  |  | 75 |  |
| (+) 23 |  |  | +(-22) |  |

| Decimal | Binary |  | Decimal | Binary |
| --- | --- | --- | --- | --- |
| -53 |  |  | -65 |  |
| (+) (-42) |  |  | +(-80) |  |

## Lab Data Sheet                TA CHECKOFF______________

4-Bit Binary Adder (Table 1)

| Cin | A3 | A2 | A1 | A0 | B3 | B2 | B1 | B0 | S3 | S2 | S1 | S0 | Co |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  |  |  |  |  |
| 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |  |  |  |  |  |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  |  |  |  |  |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |  |  |  |  |  |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |  |  |  |  |  |
| 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |  |  |  |

8-Bit Adder (Table 2):

|  | Binary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Decimal | CO | MSB 7 | 6 | 5 | 4 | 3 | 2 | 1 | LSB 0 |
| 56 | X |  |  |  |  |  |  |  |  |
| 23 | X |  |  |  |  |  |  |  |  |
| 75 | X |  |  |  |  |  |  |  |  |
| -22 | X |  |  |  |  |  |  |  |  |
| -53 | X |  |  |  |  |  |  |  |  |
| -42 | X |  |  |  |  |  |  |  |  |
| -65 | X |  |  |  |  |  |  |  |  |
| -80 | X |  |  |  |  |  |  |  |  |