Department of Electrical and Computer Engineering

COEN 212, Digital Design , Sections F and D                  Dec.13, 2018

Answer all Questions.                              All questions carry equal marks.

Exam Duration 3 hour

No books or papers are allowed.

Examiners:  Asim J. Al-Khalili and S. Das,

===============================================================

- Question  1

- Using two’s complement number system perform ( – 5 + 3 ) for a 4 bit

signed number.

- Give a binary adder circuit diagram that performs such addition.

c) Given   F(X,Y,Z) = XY + XZ’ +YZ

Implement F in NOR-NOR format, optimally.

*** Obtain optimum implementation***

Question 2

- Design a combinational circuit decoder that examines a BCD digit and displays a letter “O” if the digit was an Odd number or “E” if it was even. Note: Zero is an even number.

- Use the Display unit shown below.  Implement the circuit using minimum number of 2*1 MUXes.

a

b           c

d

e            f

g

Question 3

- Design a Half Subtractor.

- Design a Full Subtractor using two Half Subtractors.

- Using two 4 * 1 multiplexers implement the Full Subtractor

Question 4

Design a BCD adder that adds two BCD digits and produces a sum digit in BCD. You may use 4-bit binary adders for your design. Give the circuit diagram.

Question 5

Design a sequential circuit with two D flip flops A & B and two inputs E & F. If E=0, the circuit remains in the same state regardless of the value of F. When E=1 and F=1, the circuit goes through the state transition from 00 to 01 to 10 to 11, back to 00 and repeats. When E=1 and F=0, the circuit goes through the state transitions from 00 to 11, to 10 to 01, back to 00 and repeats.

Question 6

Analyze the circuit below fully. Derive the Transition Table, Excitation Table, State Diagram and the Output.  Explain the function of the circuit.

QA

TAAAAAA

FF-A

FF-B

QB

TBB

CLK

CLK