DIGITAL DESIGN COEN 212 Ted Obuchowicz/ Asim Al-Khalili

Time Allowed: 3:00 hrs. Dec 13, 2017 2 pages Answer All Questions No materials are allowed

Question 1

- Design a 3-8 decoder using NAND gates only. Mark the outputs with their corresponding binary values. (3 marks)

- Use a 3-8 decoder plus minimal extra logic gates to implement the following functions: (3 marks)

    - F1(A,B,C) =  m(0,1,2)

    - F2(A,B,C) =  m(3,4,5)

    - F3(A,B,C) =  m(5,6,7)

- Use a 4-1 MUX plus minimal extra logic gates to implement the following function: (4 marks)

F4(A,B,C,D)=A’B’ + ABC’ + AB’D

Question2

- Design a circuit that performs the following operations Y= 5A where A is an unsigned 4 bit binary number A3A2A1A0 . Give area and delay estimate of your design.    [Assume all gates have equal area and delay. A full adder has 5 gates and a half adder has 2 gates]. (6 marks)

- What size of ROM is required to implement the above multiplier? (2 marks)

Question 3

Design a Decoder that receives a BCD number and displays a L or S on a 7-segment display as shown in Fig. 1 below. If a BCD number is greater than OR equal TO 5 then L is displayed, if the number is less than 5 then S should be displayed.  Start with a truth table and derive final output in MINIMAL SOP form. Give final circuit diagram.     (8 marks)

a

b               c

d

e               f

g

a

b

c

d

e

f

g

w

x

Y

z

BCD

Fig. 1

Decoder                                                   Display

Question 4

Design a counter that counts through the sequence of odd numbers between 0 and 7,

i.e., the sequence of 1, 3, 5, 7, and back to 1.  “Use D-Flip Flop for your implementation”. (10 marks)

Question 5

- Derive the transition table of the RS Latch 	(use NOR gate). (2 marks)

- Implement the D flip-flop using the RS flip-flop, following the sequential circuit design process. (4 marks)

- Implement the JK flip-flop using a D-type flip-flop, following the sequential circuit design process.  (3 marks)

## Question 6

- Analyze the following sequential circuit and derive its state table and state diagram. (6 marks)

A

J    Q

K

J    Q

K

x

y

B

CLK

Z

- A sequential circuit has a state diagram as shown in below. Derive the equivalent state diagram with minimum number of states for the sequential circuit. Show the process. (5 marks)

1/1

1/1

0/1

0/1

0/1

1/0

1/0

1/0

1/0

1/0

1/0

0/0

0/0

0/0

0/0

0/0

000

111

001

110

010

101

011

100