--------------------------------------------------------------------------------

--

--   AMD 2910 Benchmark

--

-- Source:  AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Feb 19, 1992

--

--------------------------------------------------------------------------------

THE Functional Block MODEL

The chip has been partitioned into functional blocks which are

modeled by VHDL concurrent blocks.

The blocks are :

** muxr : Multiplexer that selects data output and tri-state-driver.

** reg_ctr : Register/Counter

** PC : Microprogram Counter

** stack_and_sp : Stack and stack pointer

** ctrl : Control Logic

Guarded concurrent blocks "reg_ctr", "PC" and "stack_and_sp" are used to

represent the sequential elements in the chip. These blocks are trigerred

by the rising edge of clock.

-----------------------------------------------------------------

VARIABLES AND SIGNALS :

File "am2910.doc" contains a general description of the chip and its ports.

This model uses the following global and local signals and variables:

*******************************************************

GLOBAL SIGNALS :

sp : stack pointer

reg_file : register file for stack

push : push signal for stack

pop : pop signal for stack

clear : clear signal for stack pointer and microprogram counter

uPC : microprogram counter

RE : register/counter

decr : decrement signal for register/counter

load : load signal for register/counter

Rzero_bar : zero detector output (corresponding to register/counter)

R_sel : Register/counter select signal to multiplexer

D_sel : Direct data input select signal to multiplexer

uPC_sel : Microprogram counter select signal to multiplexer

stack_sel : Top-of-stack select signal to multiplexer

Y_temp : Selected output from multiplexer and input to

tri-state-driver.

*******************************************************

LOCAL SIGNALS (in VHDL concurrent blocks) :

## IN BLOCK "ctrl" :

fail :  Temporary signal used in the evaluation of condition code.

-----------------------------------------------------------------

The model in file "funct_block_alg_beh2910.vhdl" contains detailed comments describing the

operation of each block. For further details, please refer to file

"am2910.doc".