-------------------------------------------------------------------------------

--

-- Floating Point Adder/Subtractor Benchmark: Main File Documentation

--

-- Source:  Patterson, David A., and Hennessy, John L.,  "Computer

--	         architecture: a quantitative approach".  San Mateo, CA: Morgank

--		 Kaufman Publishers, 1990,  Appendix A, p. 12-20

--

-- Author: Bob McIlhenny

--	   University of California, Irvine, CA 92717

--

--	   based on an original design by:

--	   Marc Rose, Intel Corporation

--

-- Written on June 9, 1993

--

-- Modified by Jesse Pan on Nov 18, 1993

--

-------------------------------------------------------------------------------

1) INTRODUCTION:

The Floating Point Adder/Subtractor is a design algorithm that performs an

operation, given the operator and two IEEE standard source operands, and

produces an IEEE standard result.

Both the two inputs and the output are represented by a sign bit, a 127-biased

integer exponent in the range 0..255, and a 23-bit vector mantissa with a

"hidden 1"(e.g. implicit representation in Patterson and Hennessy, "Computer

Architecture: a Quantitative Approach").  This is the IEEE standard, and its

representation is shown below:

______________________________

| sign | exponent | mantissa |

|______|__________|__________|

1 bit   8 bits     23 bits

------------------------------------------------------------------------------

(1.1)  FUNCTIONAL BLOCK(S)

The Adder/Subtractor has one main functional block, shown below:

** The Main functional block consists of one process: the Main process.

## Main process describes the input and output fields, including the

respective signs, exponents, and mantissas.

OPERAND 1   OPERAND 2

|           |

______v___________v______

|                       |

OPERATION --->|     FLOATING POINT    |

|    ADDER/SUBTRACTOR   |

CLOCK --->|_______________________|

|         |

v         v

RESULT    FLAGS

------------------------------------------------------------------------------

(1.2)  PORTS

INPUT AND OUTPUT PORTS:

______________________________________________________________________________

|   PORTS   | TYPE | BIT   | DESCRIPTION				      |

|           |      | WIDTH |						      |

|-----------|------|-------|--------------------------------------------------|

|    clk    |  in  |   1   | internal timing				      |

|-----------|------|-------|--------------------------------------------------|

| op1_sign  |  in  |   1   | bit sign of 1st operand			      |

|-----------|------|-------|--------------------------------------------------|

|  op1_exp  |  in  |   8   | integer exponent of 1st operand, in range 0..255 |

|-----------|------|-------|--------------------------------------------------|

| op1_mant  |  in  |  23   | 23-bit vector mantissa of 1st operand	      |

|-----------|------|-------|--------------------------------------------------|

| op2_sign  |  in  |   1   | bit sign of 2nd operand			      |

|-----------|------|-------|--------------------------------------------------|

|  op2_exp  |  in  |   8   | integer exponent of 2nd operand, in range 0..255 |

|-----------|------|-------|--------------------------------------------------|

| op2_mant  |  in  |  23   | 23-bit vector mantissa of 2nd operand	      |

|-----------|------|-------|--------------------------------------------------|

| res_sign  |  out |   1   | bit sign of result				      |

|-----------|------|-------|--------------------------------------------------|

|  res_exp  |  out |   8   | integer exponent of result, in range 0.255       |

|-----------|------|-------|--------------------------------------------------|

| res_mant  |  out |  23   | 23-bit vector mantissa of result		      |

|-----------|------|-------|--------------------------------------------------|

| operation |  in  |  ---  | signal that indicates operation, with possible   |

|           |      |       | values: add, subtract, idle                      |

|-----------|------|-------|--------------------------------------------------|

|   flags   |  out |   4   | 4-bit vector that sets status bit flags          |

|___________|______|_______|__________________________________________________|

-------------------------------------------------------------------------------

(1.3)  THE INSTRUCTION SET:

The Floating Point Adder has three instructions as follows:

** add, subtract, idle:  control performing operation and producing result

values based on input operands

-------------------------------------------------------------------------------

(1.3.1)  FUNCTIONS:

_______________________________________________________________________________

|  OPERATION  |  EFFECT UPON RESULT 				              |

|             |								      |

|-------------|---------------------------------------------------------------|

|     add     | sets result's sign, exponent, and mantissa after "adding"     |

|	      |	1st operand's addend to 2nd operand's addend		      |

|-------------|---------------------------------------------------------------|

|  subtract   | sets result's sign, exponent, and mantissa after "subtracting"|

|	      | 1st operand's addend to 2nd operand's addend	              |

|-------------|---------------------------------------------------------------|

|    idle     | maintains previous result's sign, exponent, and mantissa      |

|_____________|_______________________________________________________________|

NOTE: An addend is a 27-bit vector consisting of a "overflow" bit, the

23-bit vector mantissa, and 3 bits of precision (guard, round, and sticky

bits).

===============================================================================

2)  MODEL DEVELOPED FOR THE Floating Point Adder/Subtractor

main process -> The Adder/Subtractor is modeled as a single VHDL process

===============================================================================

3)  TESTING STRATEGY

The functions above were tested with the set of test vectors in the file:

"test.vhd"

The test vectors were chosen to exhaustively test both nominal and boundary

values of the sign, exponent, and mantissa of both source operands

Note:  The exponents used are biased-127, in other words the actual exponent

is 127 less than its representation

Also, the mantissa is represented in "hidden 1" representation

Nominal Operand:  Sign = 0 or 1,  Exponent = 1 to 254

Mantissa = any 23-bit combination of 0's and 1's, with the

exception of all 0's

The rounding conditions are tested, the rounding mode is based

on IEEE 754 floating-point standard default mode(round/even).

Boundary Operands:

Zero:  Sign = 0 or 1,  Exponent = 0,

Mantissa = 23-bit vector consisting of all 0's

Minimum Positive:  Sign = 0,  Exponent = 0

Mantissa = 23 bit-vector: 00000..01

Minimum Negative:  Sign = 1,  Exponent = 0

Mantissa = 23 bit-vector: 00000..01

Maximum Positive:  Sign = 0,  Exponent = 254

Mantissa = 23-bit vector consisting of all 1's

Maximum Negative:  Sign = 1,  Exponent = 254

Mantissa = 23-bit vector consisting of all 1's

Positive Infinity: Sign = 0,  Exponent = 255

Mantissa = 23-bit vector consisting of all 0's

Negative Infinity: Sign = 1,  Exponent = 255

Mantissa = 23-bit vector consisting of all 0's

Not a Number(NaN): Sign = 0 or 1,  Exponent = 255

Mantissa = 23-bit vector consisting of any

23-bit combination of 0's and 1's, with the

exception of all 0's

Each of the three functions was thoroughly tested for correctness and

precision.

===============================================================================

4)  STATUS OF MODELS

________________________________________________________________

|         |                  |              |        |           |

| MODEL   | TEST VECTOR USED |   SIMULATOR  | ERRORS | SYNTHESIS |

|_________|__________________|______________|________|___________|

|  main   |    		     |	   	    |  	     |	         |

| process |    test.vhd      | Synopsys 3.0 |  None  |   ----    |

|_________|__________________|______________|________|___________|