# 212: Digital Design I, week 13

## Programmable Logic Devices (PLDs)

PLDs basically store binary information in a volatile/nonvolatile device. Data is specified by designer and physically inserted (Programmed) into the device.

Any Boolean function can be expressed in Sum of Products. SOP. In turn the SOP can be implemented in an AND-OR form.

## ROM, PAL, PLA are different optimized implementations of a given circuit using the AND-OR planes.

Logic Gates

and

Programmable

switches

Inputs

(logic variables)

Outputs

( logic functions)

Programmable Logic Device as a black box

Input Buffers

and

Inverters

AND

Plane

OR

Plane

x1 x1             xn    xn

P1

Pk

Any combinational logic can be implemented with Sum of Product

which is AND-OR implementation.

f1

fm

x1 x2   xn-1 xn

Programming the AND_OR Plane

## ROM: AND Fixed, OR Programmable

## PAL:  AND Programmable, OR fixed

## PLA:  AND Programmable, OR Programmable

## FPGA: Programmable Logic Blocks, Programmable Interconnect

Example of programming the AND-OR planes

f1 (x1,x2,x3) = x1. x2   +  x1 . x3’       + x1’ .x2’. x3

f2 (x1,x2,x3) = x1. x2   +  x1’ .x2’. x3  + x3

Product

x1

x2

x3

P1

P2

P3

P4

f1

f2

Programmable Fuses

Connections

OR plane

SUM

AND plane

PLD basic cells:

The number of inputs and outputs are usually high. To avoid drawing all the inputs and outputs all the inputs to a gate is shown with one line only and the actual inputs are are shown as an intersection as shown below

AND

PLANE

PLD before programming

Programmable switches

OR

PLANE

So the previous example after programming will look like this

f1 (x1,x2,x3) = x1. x2   + x1 . x3’      + x1’ . x2’. x3

f2 (x1,x2,x3) = x1. x2   + x1’ . x2’. x3  + x1 x3

OR plane

P1

AND plane

x1

x2

x3

P2

P3

P4

f1

f2

## Programmable Array Logic (PAL )

In this category of Programmable Logic devices the AND Plane is programmable and the OR Plane is fixed

Previous Example:

f1 (x1,x2,x3) = x1 x2 x3’    + x1’ .x2 x3

f2 (x1,x2,x3) = x1’. x2’       + x1 .x2 x3

OR plane (Fixed)

P1

AND plane (Programmable)

x1

x2

x3

P2

P3

P4

f1

f2

## Programmable  Logic Array (PLA)

Both AND-Plane and the OR-Plane are programmable.

## Example:

## Implement the following functions on a PLA

Size of the Device related to:

- cost;

- speed;

- power.

?

You should choose:

- 3 variable input

- 5 product terms -> 5 AND gates

- 4 outputs -> 4 OR gates

Personality Matrix

| Product | Input |  |  | Output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | F0 | F1 | F2 | F3 |
|  | 1 | 1 | - | 1 | - | 1 | - |
|  | - | 0 | 1 | - | - | - | 1 |
|  | 1 | - | 0 | 1 | - | - | - |
|  | - | 0 | 0 | - | 1 | 1 | - |
|  | 1 | - | - | - | 1 | - | 1 |
|  | For AND gate programming |  |  | For OR gate programming |  |  |  |

1 – connection

0 – complementary

- – no connection

After programming

(* unused inputs of AND gates are connected to “1”;

** unused inputs of OR gates are connected to “0”)

When it is necessary to implement a Finite state machine, then we need a PAL or PLA that has extra circuitry such as a register and a Feedback path,

Then the following general circuitry can be added to some of the PLD devices.

f1

D      Q

Select

Enable

Clock

To AND plane

Flip-flop

Example:

Implement the Finite state Machine given by the following equations on a suitable PAL. The PAL has SR flip Flops. These equations were derived for a particular state machine using SR-Flip Flops.

Excitation Vector:

S2 = P’ Q y1,     R2 = y2,   S1 = P’ Q’ ,   R1 = Q + P

Output :

Z= y2 y1’ P Q’   ,

P & Q – are inputs

y2 & y1 are the states

z is the output

## Programming the devices:

Programming the PAL

Logic expressions first must be minimized before programming the Pal.

PALs are programmed electrically using binary patterns (as JEDEC ASCII/hexadecimal files) and by using electronic programming device from a certain manufacturer such as Data I/O Corporation's Model 60A Logic Programmer.

## User circuits are implemented in the programmable devices by configuring or programming these devices. Due to the large number of programmable switches in commercial chips; it is not feasible to specify manually the desired programming state for each switch. CAD systems are used to solve this problem.

## Computer system that runs the CAD tools is connected to a programming unit.

## After design of a circuit has been completed, CAD tool generates a file (programming file or fuse map) that specifies the state of each switch in PLD. PLD is then placed into the programming unit and the programming file is transferred from the computer system to the unit. Programming unit then programs each switch individually.

Most General of all PLDs is the FPGA or Field Programmable Gate Arrays, where, both cells of combinational logic and the routing can be programmed. Current FPGA’s are available in 10nm technology having millions of gates.

Prices range from $500 $1000

So all you need is a Personal computer, PLD CAD tool, The programming device and its software and the kind of PLD that the device accepts.

Tutorial  for  PAL can be found at

http://courses.cs.washington.edu/courses/cse370/06au/tutorials/Tutorial_PAL.htm

Schematic of an FPGA is shown below.

Logic Block

I/O Cell

Example:

Design a PLA, PAL and ROM at a gate level to realize the following sum of product functions:

X(A,B,C) = A.B + A.B.C + A.B.C

Y(A,B,C) = A.B + A.B.C

Z(A,B,C) = A + B

A

B

C

ROM, generate the minterms

X = m6, m7

Y = m6, m7

Z = m7, m6, m5, m4, m3, m2

 Fixed    programmed

ROM Implementation





















X

Y

Z

OR, generate the Functions

PAL Implementation

A

B

C

X

Y

Z

Product terms

ABC,AB,A,B

 Fixed    programmed































PLA Implementation

PLA

A

B

C

































X

Y

Z

 Fixed    programmed

Example 2

Design a function generator F=x2 where x is a 3-bit unsigned binary number.

What size of ROM is required for this generator. Show your ROM and its content.

Answer:

For the 3-bit word there is 8 possibilities, we draw the Truth Table

| x2 x1 x0 | x | f5 f4 f3 f2 f1 f0 | F |
| --- | --- | --- | --- |
| 0 0 0 | 0 | 0 0 0 0 0 0 | 0 |
| 0 0 1 | 1 | 0 0 0 0 0 1 | 1 |
| 0 1 0 | 2 | 0 0 0 1 0 0 | 4 |
| 0 1 1 | 3 | 0 0 1 0 0 1 | 9 |
| 1 0 0 | 4 | 0 1 0 0 0 0 | 16 |
| 1 0 1 | 5 | 0 1 1 0 0 1 | 25 |
| 11 0 | 6 | 1 0 0 1 0 0 | 36 |
| 111 | 7 | 1 1 0 0 0 1 | 49 |

Therefore the size of the ROM will be 8 words each of 6 bits.

It may be further minimized by noticing that f0=x0 and that f1 is=0. If we take these into account the size of the ROM is 8words of 4 bits each.

3 to 8

Decoder

Programming point

F=     f5    f4   f3   f2    f1   f0

25    24   23   22    21  20

f5 f4 f3 f2 f1 f0

0 0 0 0 0 0

0 0 0 0 0 1

0 0 0 1 0 0

0 0 1 0 0 1

0 1 0 0 0 0

0 1 1 0 0 1

1 0 0 1 0 0

1 1 0 0 01