Concordia University, Department of Electrical and Computer Engineering DIGITAL DESIGN, COEN 212 Lecturers: A. Al-Khalili , S. Das

Answer All Questions. Time Allowed 1 hr. 10min. Midterm Oct 11, 2018

Question 1

- Convert 12910  to Binary show your steps and give  the results in Binary and Hexadecimal.               (2 marks)

- Perform  the following A+B  where A and B are BCD numbers.

A=    1001   0111

B=    1001   1000            (3  Marks)

Question 2

(Use Boolean Algebra for all parts of Question 2)

a)-Simplify the following Boolean expression given by F, to obtain minimum SOP

F(W,X,Y,Z) = [XY  (X +Y’)] [( W        (W + Y)]         ( 3 marks)

b)-Calculate the frequency of operation for the circuit given below for the gate delays :

XNOR = 10 ns,  NAND = 4 ns,     OR = 8 ns,

AND = 6 ns         NOR = 7 ns,        INV= 2 ns                 (2 marks)

c)-Minimize the circuit,  and draw the  final minimized circuit

Calculate the frequency of operation of the minimized circuit.

(3marks)

a

c

F

d

a

b

## Question 3

  - Draw the timing diagram for F(A,B) = A’     B   for the following consecutive inputs. AB = 00, 10, 01, 11  (**follow these vectors in order given. Start from AB = 00** as applied at -∞). Assume the following gate delays, XOR= 6ns,  inverter = 2ns.  Use time intervals of 10ns                   (5 marks)

F

A

B

Question 4

2.a  Give minimal SOP for F(a,b,c,d) given by the following K-map

Identify the prime Implicants clearly.           ( 3 marks)

cd

ab

00

01

11

10

00     01     11    10

1      X       X      X

1       1

1                         1

X      X       X      X

2.b   Give the POS of F(A,B,C,D) = (A + CD)(B +CD)   (1 mark)

2.c   Give the minterm list of F(A,B,C,D) = B    (1 mark)

2.d   Give the minimal NOR-NOR implementation of

F(A,B,C,D) = ABC + A(D + CB)       (2 marks)