Department of Electrical and Computer Engineering

COEN 212                                                                          Oct. 15, 2015

Answer all Questions.

Exam Duration 1 hour 5min                      Examiners: Asim J. Al-Khalili,

No books / papers or electronic devices are allowed.

===============================================================

- Question  1

- Given F1 and F2 below, determine F1 . F2  and F1 + F2	          (2 Marks)

F1= AB + C               F2= A’C’ + B’C’

- Minimize the following Boolean Function:			           (2 Marks)

F(A,B,C,D)= ABC’ + BC’D’ + AC’D + ABC + BCD’ + ACD’

- Minimize F(A,B,C,D)= (AB+CD)’ + (A’B’C’D’)’                          (2 Marks)

- Question  2

1) Given   f(A,B,C) = AB +A C’ + BC

i) Implement f in NOR-NOR format                                                (2 Marks)

ii) Implement f in AND-OR-INVERT format                                  (2 Marks)

Obtain optimum implementation.

b   Give minimal POS for F(a,b,c,d) given by the K map below                  (2 marks)

| a b cd | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | X | X | X |
| 01 |  | 1 | 1 |  |
| 11 | 1 |  |  | 1 |
| 10 | X | X | X | X |

F(a,b,c,d)

- Question 3

3.a  Give the POS of  F(A,B,C,D)= (A+BCD) (B+CD)                                    (2 marks)

3.b  Give the minterm list of F(A,B,C,D)=AB                                                 (2 marks)

3.c Give the maxterm list of F(A,B,C,D)= ABC+CD+CB                               (2 marks)

3.d Minimize  F(a,b,c,d) given by the K-Map below. Give All Prime Implicant and Identify the Essential Prime Implicant. Give results in SOP                              (2 marks)

| a b cd | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | X | X | X |
| 01 |  | 1 |  |  |
| 11 | 1 | 1 |  | 1 |
| 10 | X | X | X | X |